M_FILTER for the F6FBB BBS

KE0WO @ KE0WO.#WIA.IA.USA.NA

Internet: dmadsen@iastate.edu

madsend@bvu.edu

Originally based on the M_FILTER program by OH3NWQ @ OH3RBR.FIN.EU, to

whom I owe many thanks for giving me a sample program to build from.

Without his efforts this program would not have developed as it has.

OH3NWQ:  THANK YOU!!

Features:

Optionally captures 7PLUS messages two different ways.

Works with AUTO7P

Will allow you to enforce local size/number of parts limits for 7+ messages.

Can detect @distribution swapping and automatically kill the message or

pass certain ones (base on the "To" field).

Will optionally not scan "P" type messages.

Supports WinFBB 7.00 and can theoretically run under the protected mode

Dos version of 7.00.

Includes a special 286 (and above) processor specific version for those

that run FBB on a 286 or higher machine (executable named MFLTR286.EXE).

Scanning of all messages for the content of illegal and or offensive

words that you do not feel comfortable allowing your station to

transmit.

Option to extrapolate hidden bad words.

Option to disable any rewriting of the message.

Option to specify which alert messages will be sent.

Option to hold/pass/kill messages shorter or equal to X lines.

Option to delete corrupted R: lines

Option to attempt to find duplicate/trashed messages and hold/pass/kill

them.

Can compare R: line $:bid info with other bids in the R: lines as well

as with the bid on the message.

Up to 100 strings of 15 characters or less can be scanned for.

Optionally scans the message title.

Message lines are re-formatted to 80 characters per line with facilities

to prevent a word from being chopped in the middle.

Options are configurable in the M_FILTER.CFG file.

Supports automated reporting of station activity for bulletins, arrl-

bulletins, traffic, personal messages and ACK messages.

Allows for deletion of passing WP Updates to replace them with a single

consolidated update from your station.

Those annoying-waste-of-bandwidth footers (misleading brag lines?) can

be deleted automatically: No more Received via CLOVER lines nor

signatures discussing the physics of dead animals.

Supports header standardization to several formats.

DesqView, Windows and OS/2 aware (thanks to SM0RUX for portions of the

Windows and OS/2 detection code from his CETUS unit).

Supports an alternate M_FILTER alert (great for replacing my alert with

a shorter one or one in a different language).  This is accomplished by

placing a M_FILTER.TXT text file in the \FBB directory.

Supports sending alert messages to multiple call signs.  Great for

co-sysop situations.

Will optionally remove control-G's from messages.

Can be configured to kill bulletins created by the BIDCHK program.

Runs as a TPK server as well!

How to install (FBB) DOS versions:

---------------

1. Copy M_FILTER.EXE to \FBB\BIN or any directory in the PATH.

2. Copy M_FILTER.WRD, M_FILTER.CFG, NOSWAPP.LST, REALBAD.WRD,

PERMIT.TO, STRIP.LST, to \FBB.

3. Edit the files listed in step 2.

How to install (FBB) Windows version:

---------------

1. Copy M_FILTER.DLL to \FBB\BIN or any directory in the PATH.

Do note that M_FILTER.DLL is a 286 compile by default.  There is no

MFLTR286.DLL.

2. Copy M_FILTER.WRD, M_FILTER.CFG, NOSWAPP.LST, REALBAD.WRD,

PERMIT.TO, STRIP.LST, to \FBB.

3. Edit the files listed in step 2.

How to install (TPK):

---------------

1. Copy M_FILTER.EXE to the \TPK directory (most any directory in the

PATH statement on your hard disk should work).

2. Copy M_FILTER.WRD, M_FILTER.CFG, NOSWAPP.LST, REALBAD.WRD, PERMIT.TO,

STRIP.LST, to \FBB.

3. Edit the files listed in step 2.

4. If you want to use the extrapolate feature you will have to create

the \FBB directory and but the appropriate word list there.

5. Use the command BR RXMSG SE \TPK\M_FILTER.EXE to tell TPK to run

M_FILTER.

The M_FILTER.CFG file:

---------------------

3   The first line is the error code returned to FBB when a match is

found from the words in the M_FILTER.WRD file.  0 to accept the

message, 1 or 2 to kill the message, or 3 to put the offending

message on hold.

3   The second line is the error code returned to FBB when a match is

made using the extrapolate function and the REALBAD.WRD file.  0

to accept, 1 or 2 kills, 3 holds.  Using the extrapolate features

introduces great possibilities for false alarms,  so it is not

recommended you auto kill these messages.

1   The Third line is the error code returned to FBB when a message is

found that is shorter than the length specified later in the file.

0 to accept 1 or 2 kills,  3 holds.  If you set the option to apply

this procedure to bulletins only,  then I would go ahead and kill

messages of 0 lines.

1   The fourth line is the error code returned to FBB when a message

is found that might be corrupt or a dupe.  M_FILTER looks for key

indicators that might signal a corrupt or duplicate message (having

R: lines other than at the immediate beginning of a message, having

2 or more From: lines that do not match, etc).  0 accepts the message,

1 or 2 kills,  3 holds.  I have this set to auto-kill messages.  The

dupe catching feature has been greatly improved.  I did a small study

of killed "dupes" on my system and found that out of 42 "dupes"

M_FILTER wrongfully killed only one message that it thought might be

corrupted or duplicated.  Set this option to kill at your own risk...

1   The fifth line is the error code returned to FBB when a message is

found that has had the distribution maliciously swapped.   0 to

accept the message, 1 or 2 to kill the message, or 3 to put the

offending message on hold.

3   The 6th line is the error code returned to FBB when a 7+ message

is found that exceed the limits set in the next two parameters. 0 to

accept the message, 1 or 2 to kill the message, or 3 to put the

offending message on hold.

10000

The 7th line is the 7+ message data size limit in bytes for 7+

messages (not including headers, etc).  0 Disables this function.

40  The 8th line is the limit for the number of parts that a 7+ file is

split into.  0 Disables this function.

KE0WO,N0WHI

The 9th line is the call sign of the person that will receive all

notices about offending messages.  You can have duplicates send to

more than one call sign by listing call signs separated by a comma.

KE0WO

The 10th line is the call of the BBS.

C:\FBB\MAIL.IN

The mail import file (FBB default is \FBB\MAIL.IN TPK default is

\TPK\MAIL.IN).

1   The 12th line is the flag to scan personal messages for bad words.

Set this to 0 to let personal messages to be passed without being

scanned.  A value of 1 will allow M_FILTER to scan and hold personal

messages.

1   The 13th line is the flag to scan messages from the BBS call for

offensive words or short messages.  1 enables scanning,  0 disables.

1   The 14th line is the flag to keep track of the number of ARRL

bulletins received.  This is for use with the REPORT program.

The count is maintained in the file \FBB\ARRLBUL.CNT and incremented

each time a bulletin that contains QST DE W1AW on a line is received.

The count is set back to 0 each month by the REPORT program. A 1 on

this line enables, all else disables.  If running TPK, disable.

1   The 15th line is the flag to keep track of the number of personal

message received.  This feature is also for use with the REPORT

program.  The count is maintained in the file \FBB\PERSONAL.CNT and

incremented each time a personal type message is received.

The count is set back to 0 each month by the REPORT program. A 1 on

this line enables, all else disables.  If running TPK, disable.

1   The 16th line is the flag to keep track of the number of traffic type

messages received.  This feature is for the REPORT program.

The count is maintained in the file \FBB\TRAFFIC.CNT and incremented

each time a traffic type mess is received.

The count is set back to 0 each month by the REPORT program. A 1 on

this line enables, all else disables.  If running TPK, disable.

1   The 17th line is the flag to enable M_FILTER to keep a count of

non-ARRL bulletins in the \fbb\bulletins.cnt file.

1   The 18th line is the flag to enable M_FILTER to keep a count of

ACK  (message type A) messages in the \fbb\acks.cnt file.

2   The 19th line is the flag to enable standardization of message

headers (the R: lines).  Some software will mangle the R: lines so

that they fit the format of:

R:date/time number@BBS.h.route [other info, if any]

I had my software try to undo this header destruction the best it

can by placing the headers in the format of:

R:date/time @:BBS.h.route #:number [other info, if any]

I find this format easiest to parse myself (BBS comes immediately

after the @:, message number is right after the #:, bid - $:, etc).

Some may whine about my program changing the headers.  Keep in mind

that other software changes them, so why can't mine?  Besides, this

is optional, as is the function in other software.  A 1 enables

standardization in the @:   #: format only.  A 2 will standardize

continents to two letters (ie: USA.NOAM becomes USA.NA).  A 4 will

standardize to four letter continents (ie:  USA.NA become USA.NOAM)

the best it can.  .EU is not affected by the 4 option.

1   The 20th line is a flag to specify if the BBS is to scan the R:

header lines for any matches to the word list.  A 1 enables scanning

R: lines.

1   The 21st line is a flag to enable the removal of Control-G's

("bells").   A 1 enables.

1   The 22nd line enables scanning of the message title for bad words.

This increases the processing time.  If running TPK, disable.

1   The 23rd line enables using the .001 extension on the import file

if the import file already exists.  The reason I added this option

was to eliminate the possibility of sharing conflict for those that

import bulletins through MAIL.IN instead of the regular import method

(REJECT.SYS works during the MAIL.IN imports).  (Remember to install

MAILREN if you select this option!).

1   The 24th line enables the extrapolate procedure to find hidden bad

words from the \FBB\REALBAD.WRD list.  Enabling about doubles the

time the time needed for M_FILTER to run.

1   M_FILTER will hold messages that have a number of lines equal to or

less than the number specified here.  In this case,  M_FILTER would

hold all messages with 1 line or less.  Setting to 0 disables (can't

have a message shorter than 0 lines :-).

1   Should M_FILTER attempt to find duplicate messages?

1 enables, 0 disables.

15  What types of warning messages should M_FILTER send?  To set this

parameter,  start out with the value X=0.  Then if you want to receive

warnings from M_FILTER about messages with bad words add 1 to X,  if

you want to receive messages about short messages add 2 to X, if you

want to receive messages about duplicate messages add 4 to X.  To

receive messages about maliciously swapped distributions add 8 to X.  To

receive messages about 7+ violations add 16 to X.

1   Try to prune corrupted R: lines?  This feature is designed to detect

headers after a non-header line (something that doesn't start with R:)

and drop the non-header line if headers immediately follow.  If this

option is disabled,  you will get more false alarms from the dupe

checking procedure.

Examples:

R:960119/0000Z @:PH0NE

961119/000Z @:M0DEM

R:960118/1234Z @:KE0WO.IA.USA.NA

M_FILTER drops the corrupted header from M0DEM.

R:960125/0000Z @:XX#XXX.#XXXX.XXX.XX

R:960124/0000Z @:LIDBOX.H.ROUTE [Some lengthy town name - TELINK] #:12345

$:12345_KE0WO

R:960123/0000Z @:KE0WO.#WIA.IA.USA.NA #:12345 $:12345_KE0WO

Someone's R: line is too long and the message goes through a system

that can only handle R: lines of 80 characters of less,  so it chops

the R: line at 80 characters and word wraps the leftover text as the

next line.  M_FILTER removes the $:12345_KE0WO that spills over.

Set the option to 0 to disable.

1   Is M_FILTER allowed to rewrite every message?  By default,  M_FILTER

reads every message,  then rewrites it have done its various things.

Some stations have encountered a conflict where messages are lost when

M_FILTER is installed.  I have not been able to duplicate this

problem,  which makes me think the cause of the problem is specific

to the machine and/or configuration of system that is having problems.

Until it can be determined exactly what the conflict is,  I have added

this feature as a work around for those that find this conflict.  If

M_FILTER doesn't rewrite the message,  then it can't lose it just by

reading it.  If you have problems with M_FILTER,  try setting this

option to 0.  By setting this to 0 you will lose some of the

capabilities of M_FILTER,  but the message scanning functions (the

main point of the program) will still be intact.  IF YOU EXPERIENCE

PROBLEMS AFTER INSTALLING M_FILTER SET THIS OPTION TO 0.

1   Should M_FILTER only apply the dupe screen and "short" message trap

to bulletins?  1 enables, 0 applies the procedures (if enabled) to

all messages.

C:\7PLUS\

What directory or file name should 7+ messages be stored?  Have this

line 2 characters or less (total line length or before the first

space) and the feature is disabled.  Otherwise if you end the

setting with a  \  M_FILTER assumes you are specifying a directory

and M_FILTER will store the files in that directory with the filename

contained in the 7+ message.  Otherwise,  M_FILTER assumes you

specified a file in which case M_FILTER will create and append to the

specified file name.   [In earlier versions I had been reluctant to

add this feature as I believe there are better methods to capture

7 Plus information.  With the 9602 rewrite I find that M_FILTER

should have some CPU time to burn with its increased efficiency,  so

I have added the 7 plus capture to please those that have requested

it previously.  I do not know what those people need M_FILTER to do

to be able to use AUTO7P with M_FILTER to capture messages,  so if

this isn't it,  let me know what needs to be done]

1

Should M_FILTER detect @distribution swapping of distros in the

\FBB\NOSWAPP.LST?  (The original @distro will be detected in the

To:  line.  If the @ distro doesn't match the @ field that FBB has

stored in DIRMES.SYS then the \FBB\NOSWAPP.LST is read.  If the

original @field matches a distro found in the NOSWAPP.LST file,  the

\FBB\PERMIT.TO is read.  If the message currently has a "To" field

that matches one in the PERMIT.TO file,  M_FILTER takes no action

otherwise the action specific at the beginning of the .CFG file is

taken)

1

The last line is a flag for M_FILTER to kill bulletins (and only

bulletins) spewed forth by the BIDCHK program.

The M_FILTER.WRD file:

---------------------

This file contains the list of bad-words to search for.  Maximum number

of entries is 250.  Each entry may be up to 15 characters in length and

may include any printable ASCII character except ".  Strings enclosed in

"quotes" will have to quotes removed (to allow putting spaces after the

word.  Some editor won't let you just put a space at the end of a line).

The extrapolate option and the REALBAD.WRD file:

------------------------------------------------

Now that those that are insistent upon causing problems and testing just

how far they can go have discovered that M_FILTER programs are in use.

They have also found by adding spaces, hyphens, etc between the letters

of the word they are trying to get past the sysop and the word scanner

that M_FILTER will not recognize the profane language/illegal words.

No longer.  I have added a procedure to attempt to find the hidden

bad-words that pirate stations will use so their profane message will be

forwarded all over.  The words the procedure is supposed to look for (if

the option is enabled in the M_FILTER.CFG file) is stored in the

\FBB\REALBAD.WRD file.

You will have more false alarms when this option is enabled, so I would

recommend that you only put the worst words from the M_FILTER.WRD in this

file.  Also, the longer the string - the less chance for a false alarm.

Exactly what the procedure looks for to find hidden words will remain

secret (for obvious reasons).  I'm sure that those that insist on playing

the game of getting things past M_FILTER will eventually find ways to

get stuff past.  If you care to tell me what they did to get a message

past the program I will see what I can do to adjust M_FILTER.  There is a

finite amount a program can do in this area.  There are ways to send bad

words that the program will miss,  but would be easy for a person reading

the message to notice.

The processor specific  MFLTR286.EXE:

------------------------------------

For those with 286 or higher processors,  I have included a M_FILTER

version compiled for a 286 processor (previous versions and the

M_FILTER.EXE file is compiled for a 8086 processor).  I doubt that the

286 version has much of an advantage over the 086 version.  The code is a

little smaller, maybe it runs "faster" since some of the processor

commands/calls (whatever you call it) were optimized/combined in 286 and

above processors (although the speed increase, if any, is not noticeable

to me or any other person).  If nothing else I decided that since I run a

386 for my BBS that I should at least attempt to use so of the advantages

of the processor instead of running programs I could have ran on my XT!

To use M_FILTER 286,  just copy MFLTR286.EXE to M_FILTER.EXE in the

\FBB\BIN directory replacing the previous, 8086 version of M_FILTER.

****  WARNING:  The  286 version DOES NOT VERIFY THAT YOU HAVE A 286 or

above processor.  DO NOT run it on a processor below a

286 as I haven't the faintest idea of what will happen.

Yes, I suppose that I could add the code to check, but I

am very hesitant about increasing the number of things

the program has to do after battling bugs for the past

November.  I assume that the users of my program are

intelligent enough not to install the 286 version on

their XT!

Note:  I would offer 386 and 486 versions,  but my compiler only supports

the 8086 and 80286 processors!  Sorry.

WPKILL.LST explanation

-----------------------

This is mainly for HF gateways.  Instead of having stations from all over

your area send separate WP Update messages to the global server (N6IYA at

this time).  You can put the @ destination BBS in WPKILL.LST file and wp

update messages going to that destination will be killed and a copy will

be sent to your system when the WP option is set to 1 in the

M_FILTER.WRD.  The idea behind this is that the multiple messages will

update your system and then your system will generate one consolidated

WP Update to the global server.  This is very helpful to HF stations

suddenly receiving a bunch of WP Update messages due to the changing of

the global WP server.  The WPKILL.LST file should not exist for stations

who to not have a situation that warrants this drastic of action

(ie: VHF/UHF stations).  DO NOT ABUSE THIS FEATURE!

How to strip "footers"  and taglines

------------------------------------

This version of M_FILTER allows stripping of the footers/brag lines that

some stations add to messages passing through their systems.  To use the

stripper part of this program put the file STRIP.LST in your \FBB

directory.  M_FILTER then looks through the incoming messages and if it

finds \FBB\STRIP.LST it will read the file and try to match

(case insensitive) the line in the STRIP.LST file with the first part of

each line in every message.   If a line in the message contains a portion

of a line in the strip list it will be removed.

Example:

line in STRIP.LST:

[RECEIVED VIA

matches (line in messages:)

[Received via Pactor]

[Received via HF CLOVER]

[Received via the fabulous, super-duper high speed network in Timbuktu]

you get the idea...

------- How to trap dupes or why do I sometimes have my call in

M_FILTER.WRD

I am often asked why I had the @:KE0WO.#WIA.IA.USA.NA in the M_FILTER.WRD

file.  This is to trap dupes that get their bid changed down the line and

make it back to me.  When I have the R: standardize enabled,  and R: line

scanning enabled, I know that what is left (after going through the header

manglers) of my header should contain the my call and be in the @: format.

Any duplicate messages that have my header in will then be held so I can

kill them.

Note:   If you do not have the standardize headers option enabled, you

might as well not add your call and hroute, or you will have to

add every combination of what it might look like after passing

through the other header standardizers in order for M_FILTER to

catch it.  Also, if some sysop uses the header stripper on your

header,  you are out of luck.

------------------------------- Conclusion ---------------------------------

0. M_FILTER is not fool proof or the perfect answer.  It is a useful tool

when combined with other efforts such as holding all local bulletins,

etc.  Many things can still slip past M_FILTER (such as graphically

spelling things, using double meaning phrases that use no "key" bad

words,  but can be interpreted in a perverse manner, etc.).  False

alarms and misses can not be avoided, it is a part of the game until

software can easily read and understand human languages (ie:  the

program would have to know that the English idiomatic expression

"a piece of cake"  doesn't actually refer to food, etc).  M_FILTER is

better than doing nothing, but you should not let it be the sole thing

protecting your license!

1. This software is freeware to the amateur community. It may not be sold.

It may be freely copied and distributed, but at no charge exceeding the

medium and shipping.  Commercial use of this software is prohibited

without licensing from the author.

2. No warranty is expressed or implied and incidental damages resulting

from the use of this software are solely the responsibility of the

user. While the software works fine for me, and I am willing to release

it to the amateur community, I am not willing to assume any liability

for it crashing your computer or its peripherals. (It has not done that

on any machine I have ran it on...this is just legal junk to prevent

someone from holding me responsible for being generous enough to

produce this utility as a service to the FBB community.)

3. Feedback on desired features, bugs, etc. are encouraged.  DO NOTE:  I

am a student, not a professional codesmith (yet),  so do not be

shocked when the occasional bug pops up,  but be sure to let me know

the bug exists!

4. I expect that my release will be UNALTERED down to the file name.

If I take the time to release this software, the least you can do is

not change it.

73,  hope this is useful.

Dean - KE0WO

If you have any problems, questions, comments, etc. you can contact me

via:

US Snail - Home:     Dean R. Madsen, KE0WO

P. O. BOX 303

Storm Lake, IA  50588

US Snail - College:  Dean R. Madsen, KE0WO

Wallace 7355 Lancelot

Iowa State University

Ames, IA  50013

Packet:    KE0WO @ KE0WO.#WIA.IA.USA.NA

Internet:       dmadsen@iastate.edu    (preferred)

MADSEND@BVU.EDU     (forwarded to the above account)

(Internet E-Mail will give you the 2nd fastest reply time, with phone

being 1st)

Phone:  (If you REALLY need help or just want to chat.  I tend to stay

up late (at least until 06:00Z) and sleep in late

(whenever I can :-).

Voice:     (712)  732-2021  -  Your dime.  Your best chances of catching

me at home are weekends & holidays and the

summer.

(515)  296-9474  -  The number for my dorm at Iowa State

University I am  usually around in the

evening Sunday through Thursday.

Data :    (712)  732-3767  -   14,400,N,8,1 .   Enter your call at the

prompt,  then type Y to proceed.

Fax  :    (712)  732-4773  -   14,400 US Robotics Fax modem incase you

need send written material to me in a

hurry :-).   This line is share with my

BBS's modem port                              using distinctive ringing.

Internet FTP:                  Try ftp.funet.fi somewhere in the

/pub/ham/packet/bbs/f6fbb area.

isum.iastate.edu  -  login as

anonymous.dmadsen

your email address as the password.

Experimental:              wallace05.res.iastate.edu

This will be MY computer (sitting on my

desk) connected to the internet.  This

will only be up and running while I have

my computer in my dorm room,  hooked up

to the campus ethernet and have a program

running that will accept incoming

connections.  This was discontinued May 9th

and should resume mid August.

Mailing list:

I was told that there is a mailing list for the discussion of my

programs here is the info I have:

From: SM0TSC@SM0TSC.BOLLM.AB.SWE.EU

To  : KE0WO@KE0WO.#NWIA.IA.USA.NA

I have installed a List server in my FBB and one of the LIST's is news

about KE0WO's software.. I have taken the liberty to ad you to this

mailing list..  And i hope you will send release info and other useful

info to this server. As you know i frequently visits your BBS by phone 4

downloading your latest software I will be very glad to have this info so

i can spread your fabulous soft here in EUROPE...and you can also gain

benefit by this to receive bug-reports an easy-way.. You can also mention

this server in your user-docs if you want...

To send a msg via the server:

SP KEOWO @ SM0TSC (Observe O not 0 due to not create chaos in FWD-links)

Title:AS suitable

Text follows...

etc..

-----------

For more info send a msg like this to:

SP LSERV @ SM0TSC

Title: FOO.BAR (Doesn't matter)

HELP (For help)

LISTS (For available lists)

SUBSCRIBE LIST_NAME (For subscribing to a List)